Bangladesh University of Textiles

Department of Textile Engineering Management

No. of Experiment: 02

Screening of Ring-Spinning Process Parameters Affecting Yarn Hairiness Index Using a Plackett–Burman Design (Minitab)

Complete Screening Analysis — All Replicates (H1, H2, H3)

Submitted by: Md. Hasnabul Mahdin Tahin

ID: 23010501055

Course: Textile Process Optimization Lab (TEM 310-0413)

Submitted to: Prof. Dr. Md. Syduzzaman

Date of Experiment: September 06, 2026

Date of Submission: September 13, 2026

# 1. Objective

To screen six candidate ring-spinning parameters — Spindle Speed (SS), Spinning Tension (ST), Traveler Weight (TW), Ring Diameter (RD), Yarn Count (YC) and Twist Multiplier (F) — using a 12-run Plackett–Burman (PB) design, and identify which of them significantly affect yarn Hairiness Index across three replicate measurements (H1, H2, H3), so that a focused follow-up study can target only the truly influential factors.

# 2. Theory (Brief)

- Screening design: used early in an investigation, when many factors are suspected to affect a response but a full factorial (2⁶ = 64 runs) is too costly; trades interaction resolution for a large reduction in runs.

- Plackett–Burman design: a 2-level, Resolution III orthogonal design. For 6 factors, Minitab's 12-run array gives 5 error degrees of freedom. VIF = 1.00 for every term confirms orthogonality (each main effect is estimated independently).

- Resolution III limitation: main effects are aliased with two-factor interactions, so a significant main effect could in principle be driven by an aliased interaction — hence a confirmatory follow-up design is recommended.

- Pareto chart of standardized effects: ranks factors by |t|; a Lenth reference line (2.571 at α = 0.05) separates statistically distinguishable effects from noise.

- Response: Hairiness Index (smaller-the-better). Three replicates — H1, H2, H3 — were recorded per run to study variability.

# 3. Process Parameters and Levels

| Factor | Name | Low | High | Unit |
| --- | --- | --- | --- | --- |
| A | Spindle Speed (SS) | 12000 | 16000 | rpm |
| B | Spinning Tension (ST) | 10 | 22 | cN |
| C | Traveler Weight (TW) | 2.5 | 4.0 | mg |
| D | Ring Diameter (RD) | 36 | 42 | mm |
| E | Yarn Count (YC) | 20 | 40 | Ne |
| F | Twist Multiplier (F) | 3.5 | 4.5 | – |

# 4. Experimental Design and Data (12-Run PB Array)

The table below gives the uncoded factor settings for each run together with the three Hairiness Index replicates recorded.

| Run | SS | ST | TW | RD | YC | F | H1 | H2 | H3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 16000 | 10 | 4.0 | 36 | 20 | 3.5 | 4.13 | 3.55 | 3.21 |
| 2 | 16000 | 22 | 4.0 | 36 | 40 | 4.5 | 3.79 | 4.21 | 4.90 |
| 3 | 12000 | 10 | 2.5 | 42 | 40 | 4.5 | 4.54 | 4.78 | 5.26 |
| 4 | 16000 | 10 | 4.0 | 42 | 20 | 4.5 | 4.09 | 5.21 | 3.64 |
| 5 | 16000 | 22 | 2.5 | 42 | 40 | 3.5 | 3.59 | 5.17 | 4.15 |
| 6 | 16000 | 22 | 2.5 | 42 | 20 | 3.5 | 3.29 | 3.25 | 3.17 |
| 7 | 12000 | 10 | 4.0 | 42 | 40 | 3.5 | 4.49 | 4.17 | 5.17 |
| 8 | 12000 | 10 | 2.5 | 36 | 20 | 3.5 | 4.15 | 5.66 | 4.78 |
| 9 | 16000 | 22 | 4.0 | 36 | 20 | 4.5 | 5.17 | 3.89 | 5.64 |
| 10 | 12000 | 22 | 4.0 | 36 | 40 | 3.5 | 4.63 | 2.17 | 4.17 |
| 11 | 12000 | 22 | 2.5 | 36 | 20 | 4.5 | 4.97 | 4.64 | 5.28 |
| 12 | 12000 | 10 | 2.5 | 36 | 40 | 4.5 | 3.77 | 2.64 | 5.64 |

Mean Hairiness Index across the three replicates ranged from 3.24 (Run 6) to 4.96 (Run 11), indicating a wide response spread suitable for screening.

# 5. Screening Analysis (Factorial Regression, all replicates)

A linear main-effects model was fitted separately for each replicate via Stat > DOE > Screening > Analyze Screening Design. Results (ANOVA, regression equation) are summarised below for H1, H2 and H3.

## Response H1

| Source | DF | Adj SS | F-Value | P-Value |
| --- | --- | --- | --- | --- |
| Model | 6 | 1.645 | 0.74 | 0.639 |
| SS (A) | 1 | 0.049 | 0.13 | 0.729 |
| ST (B) | 1 | 0.078 | 0.21 | 0.664 |
| TW (C) | 1 | 0.273 | 0.74 | 0.429 |
| RD (D) | 1 | 0.166 | 0.45 | 0.532 |
| YC (E) | 1 | 1.062 | 2.88 | 0.150 |
| F | 1 | 0.017 | 0.05 | 0.839 |
| Error | 5 | 1.842 |  |  |
| Total | 11 | 3.487 |  |  |

Model summary: S ≈ 0.607, R-sq ≈ 47.2%, R-sq(adj) ≈ 0% (negative, i.e. no real explanatory power) — model not significant (P = 0.639).

Regression equation (uncoded units): H1 = 2.70 + 0.000032·SS − 0.0135·ST + 0.201·TW + 0.0392·RD − 0.0298·YC + 0.036·F

No factor reaches significance at α = 0.05. Yarn Count (E) has the lowest P-value (0.150) and the largest share of explained variation, making it the most influential — though inconclusive — factor for this replicate.

## Response H2

| Source | DF | Adj SS | F-Value | P-Value |
| --- | --- | --- | --- | --- |
| Model | 6 | 5.239 | 0.65 | 0.693 |
| SS (A) | 1 | 2.159 | 1.61 | 0.260 |
| ST (B) | 1 | 1.086 | 0.81 | 0.409 |
| TW (C) | 1 | 0.037 | 0.03 | 0.874 |
| RD (D) | 1 | 0.249 | 0.19 | 0.684 |
| YC (E) | 1 | 0.715 | 0.53 | 0.497 |
| F | 1 | 0.992 | 0.74 | 0.429 |
| Error | 5 | 6.691 |  |  |
| Total | 11 | 11.930 |  |  |

Model summary: S = 1.157, R-sq = 43.9%, R-sq(adj) = 0.00% — model not significant (P = 0.693).

Regression equation (uncoded units): H2 = 5.12 − 0.000212·SS − 0.0501·ST − 0.074·TW + 0.048·RD + 0.0244·YC − 0.288·F

Again no factor is significant. Spindle Speed (A) has the lowest P-value (0.260) and the largest sum of squares, standing out as the most influential factor for this replicate.

## Response H3

| Source | DF | Adj SS | F-Value | P-Value |
| --- | --- | --- | --- | --- |
| Model | 6 | 10.037 | 3.06 | 0.120 |
| SS (A) | 1 | 0.004 | 0.01 | 0.935 |
| ST (B) | 1 | 0.760 | 1.39 | 0.292 |
| TW (C) | 1 | 0.203 | 0.37 | 0.569 |
| RD (D) | 1 | 0.059 | 0.11 | 0.756 |
| YC (E) | 1 | 0.811 | 1.48 | 0.278 |
| F | 1 | 8.201 | 14.98 | 0.012 |
| Error | 5 | 2.737 |  |  |
| Total | 11 | 12.775 |  |  |

Model summary: S = 0.740, R-sq = 78.6%, R-sq(adj) = 52.9% — model not significant overall (P = 0.120), but one term is.

Regression equation (uncoded units): H3 = 5.21 + 0.000009·SS − 0.0419·ST − 0.173·TW − 0.0233·RD + 0.0260·YC + 0.827·F

Twist Multiplier (F) is the only factor significant at α = 0.05 (P = 0.012), accounting for 8.20 of the 10.04 model sum of squares (≈ 82% of explained variation) — by far the dominant effect in this replicate.

# 6. Comparative Significance Across Replicates

| Factor | P-value H1 | P-value H2 | P-value H3 | Lowest-P Replicate |
| --- | --- | --- | --- | --- |
| SS (A) | 0.729 | 0.260 | 0.935 | H2 |
| ST (B) | 0.664 | 0.409 | 0.292 | H3 |
| TW (C) | 0.429 | 0.874 | 0.569 | H1 |
| RD (D) | 0.532 | 0.684 | 0.756 | H1 |
| YC (E) | 0.150 | 0.497 | 0.278 | H1 |
| F | 0.839 | 0.429 | 0.012* | H3 |

*Significant at α = 0.05.

No single factor is consistently significant across all three replicates, which is expected given the Resolution III design and only 5 error degrees of freedom per model. Twist Multiplier (F) is the only factor to cross the significance threshold, and only in replicate H3. Yarn Count (E) trends strongest in H1, and Spindle Speed (A) trends strongest in H2. Together, F, A and E emerge as the recurring 'candidates of interest', while Spinning Tension (B), Traveler Weight (C) and Ring Diameter (D) show consistently high P-values across all three replicates and can be treated as the 'trivial many'.

# 7. Alias Structure (Note)

Because the design is Resolution III, every main effect is aliased with a specific combination of two-factor and higher-order interactions (e.g. A is aliased with a linear combination of BC, BD, BE, BF, CD, … terms, each with a coefficient of ±0.33). This means an apparently significant main effect — such as F in replicate H3 — could, in principle, be partly driven by an aliased interaction rather than a pure main effect. The full alias listing is identical for all three replicates (it depends only on the design matrix, not the response) and is omitted here for brevity; it is available in the Minitab session output.

# 8. Recommended Follow-Up Study

- Run a focused, higher-resolution design (full 2³ factorial, Response Surface, or Taguchi) on Twist Multiplier, Spindle Speed and Yarn Count — the three recurring candidates of interest — to obtain de-aliased, replicate-stable estimates.

- Fix Spinning Tension, Traveler Weight and Ring Diameter at convenient mid-range production settings, since they showed consistently high P-values (low influence) across all three replicates.

- Increase replication (3–5 readings per run) in the follow-up study to reduce error variance, since the current 12-run screening models explain little variance on new data (R-sq(pred) ≈ 0% throughout) and results vary noticeably between replicates.

# 9. Conclusion

A 12-run Plackett–Burman screening design was used to evaluate six ring-spinning parameters against yarn Hairiness Index, with three replicate measurements (H1, H2, H3) analysed independently. Twist Multiplier was the only factor to reach statistical significance (P = 0.012), and only in replicate H3, where it explained roughly 82% of the model's variation. Spindle Speed and Yarn Count showed the strongest (though not significant) trends in replicates H2 and H1 respectively, while Spinning Tension, Traveler Weight and Ring Diameter were consistently unimportant across all three replicates. The variability between replicates, combined with the design's Resolution III alias structure, means these results should be treated as screening indicators rather than final conclusions. A focused confirmatory study on Twist Multiplier, Spindle Speed and Yarn Count is recommended before finalising production settings.